Schedule A

Requirements for Plans

# 1.	General

  - Plans should be drawn to an appropriate scale shown on the drawings.

  - Plans should be drawn with clarity.

  - Plans should indicate a north point.

  - All plans shall be consistent with each other.

2.	Survey plans are to indicate:

- Existing buildings, structures and features of the site;

- Topography (spot levels, contours) including that of adjoining property where relevant;

- Natural drainage of the site;

- Any easements or rights of way;

- Significant existing vegetation, indicating its location on the site, type and spread;

- Location, height and use of any adjoining buildings or structures such as swimming pools; and

- Features of streets immediately adjoining or within the property, including poles, kerbs, crossings and pits.

3.	Site plans are to identify the location of the following:

- Proposed and existing buildings;

- Existing significant trees, indicating whether they will be retained or removed;

- Paved areas;

- Landscaped areas;

- Driveway entry and/or exit;

- Garbage storage areas;

- On-site detention tanks;

- Letter boxes;

- Private open spaces; and

- Where privacy is an issue in the proceedings, the location of windows of the adjoining property and the subject proposal.

4.	Floor plans are to indicate:

- Room names, area and dimensions;

- The location of windows and doors;

- The levels of floors, terraces and the like to Australian Height Datum (AHD);

- Wall construction; and

- Spot levels of natural ground to AHD.

5.	Elevations are to indicate:

- Elevations of all sides of the building or structure;

- Outline of existing buildings;

- Materials and finishes to be used in construction;

- Location of adjoining buildings showing address, height, setbacks and other relevant features;

- Proposed window size, sill height and location; and

- Height of eaves, ridge and floor levels to AHD.

6.	Sections are to indicate:

- Appropriate number and location;

- Section line and location on plan;

- Room names;

- Adequate representation of ground level;

- Areas of cut and/or fill; and

- Height of levels to AHD.

7.	Landscape plans are to:

- Be consistent with other plans tendered to the court with respect to the height, size and location of buildings;

- Indicate the location, species, height and spread of significant existing trees, indicating whether they will be retained or removed;

- Indicate the location of any additional planting to be carried out including species names, spread, height and other features; and

- Indicate the location of significant retaining walls or other structures.

- Indicate finished relative levels of all major surfaces.

8.	Overshadowing plans are to:

- Be based on true north;

- Indicate the location and nature of existing and/or proposed fencing, with the shadows projected;

- Indicate horizontal and vertical impact, including any impact from any substantial wall;

- Provide a table of compliance and non-compliance with known criteria (such as a development control plan, a State environmental planning policy or Australian Model Code for Residential Development (AMCORD)); and

- Make appropriate allowance for the topography.