Use of photomontages

The following requirements for photomontages proposed to be relied on as or as part of expert evidence in Class 1 appeals will apply for proceedings commenced on or after 1 October 2013. The following directions will apply to photomontages from that date:

Requirements for photomontages

- Any photomontage proposed to be relied on in an expert report or as demonstrating an expert opinion as an accurate depiction of some intended future change to the present physical position concerning an identified location is to be accompanied by:

Existing Photograph.

- A photograph showing the current, unchanged view of the location depicted in the photomontage from the same viewing point as that of the photomontage (the existing photograph);

- A copy of the existing photograph with the wire frame lines depicted so as to demonstrate the data from which the photomontage has been constructed. The wire frame overlay represents the existing surveyed elements which correspond with the same elements in the existing photograph; and

- A 2D plan showing the location of the camera and target point that corresponds to the same location the existing photograph was taken.

Survey data.

- Confirmation that accurate 2D/3D survey data has been used to prepare the Photomontages. This is to include confirmation that survey data was used:

  - for depiction of existing buildings or existing elements as shown in the wire frame; and

  - to establish an accurate camera location and RL of the camera.

- Any expert statement or other document demonstrating an expert opinion that proposes to rely on a photomontage is to include details of:

- The name and qualifications of the surveyor who prepared the survey information from which the underlying data for the wire frame from which the photomontage was derived was obtained; and

- The camera type and field of view of the lens used for the purpose of the photograph in (1)(a) from which the photomontage has been derived.